# Solar Irradiation and Energy Data Sheet

## Bogotá Savanna (Sabana de Bogotá), Colombia

| Parameter | Value / Range | Units | Observations / Notes |
| --- | --- | --- | --- |
| Geographic Location | 4°35'N, 74°04'W | — | Located near the equator in the Eastern Cordillera of the Colombian Andes. Near-equatorial position ensures consistent solar angles and day lengths year-round. |
| Average Altitude | ~2,600 | m a.s.l. | High altitude reduces atmospheric thickness, lowering air mass and increasing potential solar irradiance at the surface. |
| Climate Type | Tropical Highland (Köppen: Cwb) | — | Mild temperatures year-round, bimodal rainfall pattern (two rainy and two dry seasons), and persistent cloud cover during rainy seasons. |
| Average Global Horizontal Irradiance (GHI) | 4.0 – 5.0 | kWh/m²/day | Moderate-to-good irradiance for a tropical location. High altitude and equatorial position provide strong potential, partially reduced by cloudiness (~4.5 average). |
| Annual GHI | ~1,600 – 1,700 | kWh/m²/year | Derived from daily average (4.5 × 365 ≈ 1,643). Comparable to parts of southern Europe, making the region competitive for solar energy. |
| Direct Normal Irradiance (DNI) | 3.0 – 3.5 | kWh/m²/day | Reduced by frequent cloud cover. This makes concentrating solar power (CSP) less viable, while flat-plate PV systems remain the preferred technology. |
| Diffuse Horizontal Irradiance (DHI) | 1.5 – 2.0 | kWh/m²/day | High diffuse fraction (~35–45% of GHI) due to cloud scattering. Flat-plate PV panels capture diffuse radiation effectively from all sky directions. |
| Average Solar Peak Hours | ~4.5 | h/day | Equivalent to daily GHI value. Represents hours of equivalent full irradiance (1,000 W/m²) per day — key parameter for PV system sizing. |
| Average Air Temperature | 13 – 14 | °C | Cool and stable year-round due to altitude. Beneficial for PV: silicon panels lose ~0.4–0.5% efficiency per °C above 25°C, so cool conditions preserve rated output. |
| PV System Efficiency (Typical) | 15 – 20 | % | Standard commercial monocrystalline or polycrystalline silicon panels. Cool operating temperatures help maintain efficiency close to rated values. |
| Estimated PV Energy Yield | 1,200 – 1,400 | kWh/kWp/year | Calculated as: PSH × 365 × PR (0.75–0.80). Example: 4.5 × 365 × 0.78 ≈ 1,281 kWh/kWp/year. Sufficient for economically viable solar installations. |
| Seasonal Variability | Low to Moderate | — | Bimodal rainfall causes ±15–20% GHI variation between seasons. Much lower variability than temperate regions due to consistent equatorial day length. |
| Main Limiting Factors | Cloud cover, atmospheric humidity, seasonal rainfall | — | Persistent cloud cover in rainy seasons (Mar–May, Sep–Nov) is the primary limitation, reducing DNI significantly. Low dust accumulation is an advantage over arid regions. |
| Suitability for Solar Energy | Moderate to Good | — | Equatorial location, high altitude, cool temperatures, and acceptable GHI make the Bogotá Savanna well-suited for flat-plate PV systems. Main engineering challenge is designing for high diffuse radiation and seasonal cloud variability. |

Conclusion: The Bogotá Savanna is well-suited for photovoltaic solar energy. Its cool temperatures preserve panel efficiency, its near-equatorial position minimizes seasonal fluctuation, and its annual GHI is competitive globally. Flat-plate PV is the recommended technology given the high diffuse radiation fraction.